## UART Transmitter Module

This module is all about getting a basic UART transmitter up and running. It uses the 8N1 format, which stands for:

- 8 data bits

- No parity bit

- 1 stop bit

It sends serial data at 9600 baud, with the timing derived from a 12 MHz oscillator. The module runs on a simple state machine and includes some bonus visuals—like blinking RGB LEDs—to make sure you can tell it’s doing something!

## 1. Understanding the Code

### Top Module: The Brain of the Operation

The top module pulls together several components:

- A 12 MHz internal clock

- A clock divider to generate a 9600 Hz signal

- The UART transmission logic

- A small RGB LED control system

Here’s how each part plays its role:

- Clock Generation: It starts with an internal 12 MHz oscillator.

- Clock Division: That frequency is divided by 1250. Since UART clocks need to toggle every half-period, the actual toggle happens every 625 cycles. This results in a clean 9600 Hz clock, which matches our baud rate.

- UART Transmission: The transmitter keeps sending the character 'D' over and over using the 8N1 format.

- LED Blinking: The RGB LEDs are connected to a counter. Different bits of this counter toggle the LEDs, so they blink at regular intervals, giving you a visual confirmation that the system is active.

In short, this module handles timing, transmits a test character, and blinks LEDs—all in sync.

### uart_tx_8n1 Module: How Data is Sent

The actual transmission logic lives in the uart_tx_8n1 module. It uses a Finite State Machine (FSM) to go through the steps of sending each bit of the UART frame.

### Baud Rate Generator: Getting to 9600 Baud

Here’s how it creates the correct timing signal:

- A counter increases with every tick of the 12 MHz clock.

- When it hits 1249, it resets and toggles a signal called baudclk_en.

- Since this toggle happens every 1250 cycles, it results in a 9600 Hz signal, which matches the required UART baud rate.

### State Machine Breakdown: Step-by-Step Transmission

The FSM moves through different states to send a full UART frame:

#### 1. IDLE (STATE_IDLE)

- When senddata = 1:

  - The module moves to STATE_STARTTX

  - The byte to be sent (txbyte) is loaded into a buffer called buf_tx

  - txdone is cleared, meaning transmission is now in progress

- If senddata = 0, it stays idle:

  - txbit stays high (UART line idles high)

  - txdone stays low

#### 2. START BIT (STATE_STARTTX)

- txbit is set to 0 to indicate the start of transmission

- The system then moves to STATE_TXING

#### 3. SENDING DATA (STATE_TXING)

- As long as bits_sent < 8:

  - The least significant bit (LSB) of buf_tx is sent out through txbit

  - buf_tx is shifted right to prepare the next bit

  - bits_sent is increased by 1

#### 4. STOP BIT (STATE_TXDONE)

- After all 8 data bits are sent:

  - txbit is set to 1 (stop bit)

  - bits_sent is reset

  - State changes to STATE_TXDONE

#### 5. DONE → BACK TO IDLE

- In STATE_TXDONE, the system:

  - Sets txdone = 1 to indicate the frame is fully transmitted

  - Moves back to STATE_IDLE, ready for the next byte

## 2. System Architecture

### Block Diagram

This diagram shows how the modules are wired together: oscillator, clock divider, UART logic, and LED driver.

### Circuit Diagram

This shows how the components are connected physically—great for when you're wiring it up on your FPGA board.

## 3. Synthesis & Programming

Once the code is ready, here’s how to test everything:

### Step 1: Clone the Repo

bash

CopyEdit

git clone

### Step 2: Build the Bitstream

bash

CopyEdit

make build

This command compiles the project and creates top.bin, which you’ll load onto your FPGA.

### Step 3: Flash It to the FPGA

bash

CopyEdit

sudo make flash

This uploads the design onto your FPGA board.

### Step 4: Open the UART Terminal

bash

CopyEdit

sudo make terminal

Once this runs, you should see the letter 'D' appearing repeatedly. That’s your UART transmission in action—sending data at 9600 baud, just as expected.